🍜 Product Configuration Tutorial

Multi-Level BOM & Routing for Canton Noodles 300g (PROD-001)
Mapped to Standard Operating Procedure (S.O.P.)

In professional production environments (such as ERP structures), manufacturing is modeled using a Multi-Level Bill of Materials (BOM) to separate processing stages from final packaging. Instead of combining everything into one flat list, the production of Canton Noodles 300g is split into two relational levels:

The Base Product (The Pack/Piece): Represents the single retail 300g pack of noodles. This handles processing steps 1 to 6 (Blanching, Frying, Pre-Drying, Packaging).

The Parent Product (The Box/Case): Represents the master shipping box containing 18 retail packs. This handles step 7 (Boxing & Palletizing) and consumes the base pack as a sub-assembly ingredient.

# Part 1: Configuring the Base Product (Pack/Piece)

First, register the single retail unit of Canton Noodles in the Finished Goods registry.

Product Registration Details:
• SKU Code: PROD-001
• Product Name: PROD-001 Canton Noodles 300g Pack
• Base Unit of Measure (UoM): PCS (or PACK)
• Version Name: v1.0 - Single Pack Form (Active Status)

Route Steps & Processing Durations (Steps 1 to 6):

| Step | Work Station | Operation Name | Run Time (per Unit) | Setup Time |
| --- | --- | --- | --- | --- |
| 1 | Blanching & Cooling Station (1st & 2nd Tubs) | Blanching & Cooling | 00:00:45 | 00:15:00 |
| 2 | Fixing & Molder Prep Table | Noodle Fixing & Prep | 00:00:15 | 00:05:00 |
| 3 | Canton Frying Station (5 Tubs @ 140-160°C) | Frying & Oil Draining | 00:02:00 | 00:30:00 |
| 4 | Pre-Drying & Blower Conveyor Line | Tunnel Drying & Blowing | 00:00:30 | 00:10:00 |
| 5 | Weighing & Inspection Station | Weighing & Quality Audit | 00:00:10 | 00:00:00 |
| 6 | Canton Packaging & Sealing Machine | Automated Packing & Cutting | 00:00:03 | 00:20:00 |

Bill of Materials (BOM) Ingredients per Individual Pack:

| Route Step | Ingredient Name | Qty Required | Wastage % |
| --- | --- | --- | --- |
| 1. Blanching | Wheat Flour (Canton Grade) | 0.220 kg | 1.50% |
| 3. Frying | Refined Frying Palm Oil | 0.090 kg | 3.00% |
| 6. Packaging | Printed Canton 300g Polybag | 1.000 PC | 2.00% |

# Part 2: Configuring the Parent Product (Box/Case)

Next, register the Case Box which will consume the retail packs as a sub-assembly.

Product Registration Details:
• SKU Code: PROD-001-BOX
• Product Name: PROD-001-BOX Canton Noodles 300g Box (18 Packs)
• Base Unit of Measure (UoM): BOX (or CASE)
• Parent Product: PROD-001 Canton Noodles 300g Pack (Select from list)
• Version Name: v1.0 - Box of 18 Packs (Active Status)

Box Processing Route Step:

| Step | Work Station | Operation Name | Run Time (per Unit) | Setup Time |
| --- | --- | --- | --- | --- |
| 1 | Boxing & Palletizing Area (25 Boxes/Pallet) | Boxing & Pallet Stacking | 00:01:30 | 00:05:00 |

Bill of Materials (BOM) Ingredients per Carton Box:

| Route Step | Ingredient Name | Qty Required | Wastage % |
| --- | --- | --- | --- |
| 1. Boxing | PROD-001 Canton Noodles 300g Pack (Child Product) | 18.000 PCS | 0.00% |
| 1. Boxing | Canton 300g Master Shipping Box (Packaging Material) | 1.000 PC | 0.50% |

# Part 3: Cost Rollup & Margin Simulation

When you simulate production runs or calculate margins for PROD-001-BOX:

Dynamic Sub-Assembly Rollup: The system aggregates the raw material cost (Wheat Flour, Palm Oil, Polybag) and the conversion costs of the 6 processing line workstations for all 18 packs.

Parent Costs Added: It adds the cost of the Master Shipping Box and the conversion overhead rate of the Boxing & Palletizing workstation.

Margin Calculation: Inputting a Target Selling Price (e.g. ₱600.00 per Box) will calculate the gross profit margin, net profit per box, and break-even batch volume based on setup and run times.

End of Tutorial. Nothing follows.